GEOLOGICAL SUMMARY — DOCUMENT 02.1 (UPDATED)  |  DATA ROOM

Geology & Mineralisation

Nairn Ni Cu PGE Project — Sudbury Igneous Complex, SW Footwall

# Regional Geological Setting

The Sudbury Igneous Complex (SIC) is a ~1.85 Ga impact melt sheet, one of the world's most prolific Ni-Cu-PGE metalliferous districts, accounting for roughly 23% of current global nickel production. The SIC consists of Norite (Lower Zone), Quartz Gabbro (Middle Zone), and Granophyre (Upper Zone), underlain by footwall Sudbury Breccia and overlain by the Whitewater Group metasediments.

Ni-Cu-PGE mineralisation at Sudbury occurs as contact-type deposits along the SIC lower contact and as Offset Dyke deposits (SODs) that radiate outward into the footwall. Offset Dyke-type deposits are the primary target at Nairn.

# Sudbury Offset Dykes — Deposit Model

SODs are quartz diorite intrusions extending outward from the SIC. Mineralisation within and adjacent to SODs includes: sharp-contact massive sulphide at dyke margins; disseminated to net-textured sulphide in marginal phases; and vein-type sulphide in fractures. Grades increase with depth in all producing SOD systems. The new 8.45% Ni surface sample, with ~17% total sulphur, is diagnostic of massive sulphide — the highest-grade ore type in SOD deposits.

# Nairn Property Geology

The Nairn property is positioned 5–10 km WSW of the Sudbury Camp's Worthington Dyke and Mystery Dyke — structural corridors hosting Victoria, McIntyre, AER, Worthington, and Totten mine complexes. The property covers a gabbro and diabase-dominated assemblage within the SIC footwall, exhibiting sudburite alteration typical of mineralised SIC contacts.

Key structural features: (1) the Worthington Offset Dyke SW extension, whose surface trace is mapped onto the western claims; and (2) the Mystery Dyke SW extents, known surface outcrop within 2 km of the NE boundary.

# Mineralisation

Sulphide mineralogy includes pyrrhotite (dominant), chalcopyrite, pentlandite, pyrite, and marcasite. Sulphide textures range from disseminated (5–20%) to massive (50–75% at Berry Shaft, ~17% S in new sample). The confirmation of pentlandite (primary Ni mineral in Sudbury deposits) at multiple locations, plus the now-documented gold credit (2.38 g/t Au in the new sample), substantially enriches the polymetallic value proposition.

# Sampling and Assay Results — Updated

★ NEW HEADLINE RESULT — OMITTED FROM PRIOR PRESENTATION Surface massive sulphide sample: 8.45% Ni · 6.63% Cu · 0.45% Co · 2.38 g/t Au + strong Pd–Pt credits ~17% total sulphur confirms massive sulphide (not disseminated). The gold credit (2.38 g/t Au) is a newly identified value driver not present in prior sampling. This is the highest-grade result on the entire property and was inadvertently omitted from the September 2025 presentation.

| Location | Method | Ni (%) | Cu (%) | Co (%) | Notes / PGE |
| --- | --- | --- | --- | --- | --- |
| Surface massive sulphide | Grab | 8.45 | 6.63 | 0.45 | 2.38 g/t Au + Pd-Pt, ~17% S ★ |
| Adit face | Face | 5.56 | 1.63 | 0.54 | — |
| Berry Shaft muck | Grab (Mustang) | 1.28 | 5.42 | 0.142 | 213 ppb Pt, 254 ppb Pd |
| Barry South Pit | Grab | — | 1.59* | 0.21 | 219 ppb PGE |
| Trench / chip | Channel | 1.16 | — | — | 3.00 m interval |
| Drill intercept | DDH | 1.28 | — | — | 3.00 m interval |
| Grab — Zone A | Grab | — | 2.49* | 0.12 | 271 ppm PGM |
| Grab — Zone B | Grab | — | 2.95* | 0.09 | 240 ppm PGE |
| Cu-rich zone | Grab | 1.00 | 19.00 | — | Massive sulphide zone |

★ The surface massive sulphide sample (8.45% Ni) was collected in 2024 and was omitted from the September 2025 presentation. ~17% S confirms massive sulphide texture. Au (2.38 g/t) is a newly identified value driver. Values marked (*) are combined Cu+Ni%. All assays pre-NI 43-101 QA/QC unless stated.

# Geophysics

- 2001–02 — Ground magnetics and VLF. Identified magnetic lows coinciding with mafic intrusions; VLF lineaments interpreted as structural corridors.

- 2012–13 — Titan-24 DC-IP (19.2 km, 8 lines). High-chargeability zones and conductivity lows aligned with geological contacts. Priority drill targets defined. Never tested.

- 2022 — VTEM anomaly investigated by Vale. Unexplained. Ground EM follow-up recommended but not completed.

# Priority Drill Targets

## Priority 1 — Worthington Offset Dyke SW Extension (Berry Shaft Depth)

Down-dip projection of the Worthington Offset Dyke beneath Berry Shaft. Surface samples (8.45% Ni, 5.56% Ni) confirm the system. Embayment zones hosting economic deposits (Totten, Victoria) are expected at 300–1,000 m depth.

## Priority 2 — Alanen Showing / Mystery Dyke NE Margin

Historic trenches exposed abundant pyrrhotite-chalcopyrite in diabase. Mystery Dyke within 2 km. IP data shows chargeability anomalies in this sector — never drill-tested.

## Priority 3 — Barry South Pit and Gold-Copper Zone

New data indicates a gold-enriched copper zone (2.38 g/t Au, 6.63% Cu) coinciding with the massive sulphide horizon. This Au credit substantially increases the economic potential of the near-surface zone and warrants targeted sampling and drilling.

## Recommended Phase 1 Program

- IP geophysics: 2,000–3,000 m of new dipole-dipole lines over Targets 1 and 2.

- Ground EM (TDEM) follow-up: 600+ line-metres over the VTEM anomaly.

- 5–8 diamond drill holes (300–500 m depth) testing all three priority targets.

- Full QA/QC sampling of all accessible exposures, including twin-sampling of new 8.45% Ni sample location.

- Detailed 1:5,000 geological mapping across the central claim block.

KEY REFERENCES

GeologyOntario Assessment Files: IDs 20000022000, 20000022112, 20000021235, 20000008521. Vale Canada Ltd. internal report (2022). Sino Minerals Corp. Titan-24 IP survey report (2013). Mustang Minerals Ltd. sampling report (2002-03). Vision Exploration field report (2024). Updated to include 2024 massive sulphide sample (previously omitted).