EXPLORATION HISTORY — DOCUMENT 03.1  |  DATA ROOM

Exploration History

Nairn Ni Cu PGE Project — Summary of Past Work

# Timeline of Work Programs

The following table consolidates all known exploration work on the Nairn property from pre-1940 historical workings through to the 2023–2024 field programs. Full GeologyOntario assessment files are listed in the references section.

| Period | Operator | Work Completed & Key Results |
| --- | --- | --- |
| Pre-1940 | Unknown / Historic | Berry Shaft developed (two water-filled shafts + ore dump). Barry South Pit developed. Historical pits and trenches including Alanen Showing. Limited channel sampling in trenches. Target commodities: Cu, Ni, Co, PGE. Hosts: gabbro and diabase with pyrrhotite, chalcopyrite, pentlandite, pyrite. |
| 2001–2003 | Mustang Minerals Ltd. | Ground magnetics and VLF survey (A. Barry). Grab samples from Berry Shaft muck pile: 5.42% Cu, 1.28% Ni, 0.142% Co, 213 ppb Pt, 254 ppb Pd — among the highest-grade surface results in the area. Trench grabs: 0.99% Cu, 0.64% Ni, 0.083% Co, 27 ppb Pt, 96 ppb Pd. Barry South Pit: 1.59% Cu+Ni, 0.21% Co, 219 ppb PGE. OGS composite at Berry Shaft: 2.92% Cu, 0.41% Ni. |
| 2012–2013 | Sino Minerals Corp. | Titan-24 DC-IP survey: 19.2 km across 8 lines (pole-dipole, 100m dipoles) mapping chargeability and resistivity. High chargeability zones identified (e.g., IP8 at 1100S/L400W). Conductivity lows aligned with geological contacts interpreted as mafic dike contacts and NE-trending gabbro sills — priority drill targets defined. Integration with prior 2011 Titan-24 IP data. Concurrent geochemical survey identified mineralised occurrences. |
| 2022 | Vale Canada Ltd. | Beep-mat reconnaissance over VTEM anomaly (potentially Worthington Offset related). No conductive/magnetic sources detected at surface. 0.25 km² mapped; focus on metasedimentary and metagabbroic rocks. 26 grab samples (21 assayed for whole-rock + PGE + 33 elements) — no anomalous Ni, Cu, or PGE results; only trace pyrite. No quartz diorite (key SOD indicator) located. Ground EM survey over 600m lines recommended but not executed. |
| 2023 | Steven Anderson / Vision Exploration | One field day on claim 548887: located historic N-S trenches (trending E-W) at Alanen Showing. Abundant pyrrhotite and chalcopyrite in diabase confirmed. 4 grab samples collected for Cu, Ni, PGE assays (results not yet detailed in filed report). |
| 2024 | Steven Anderson / Vision Exploration | One field day on claim 214048: Berry Shaft and ore dump located. 4 grab samples from gabbro with 50–75% sulphides sent for assay (results not yet detailed). Channel sampling evidence in historical trenches identified but no new channel sampling conducted. |

# Summary Interpretation

## What has been established

- Multiple Ni-Cu-Co-PGE surface showings confirmed across the property (Berry Shaft, Alanen, Barry South Pit).

- High-grade samples (up to 5.56% Ni in adit; 5.42% Cu + PGE from muck) confirm system prospectivity.

- Chargeability anomalies from two independent IP programs (2011/2013) coincide with geological contacts — these are the defined drill targets.

- A 3.00 m drill intercept at 1.28% Ni confirms the system continues below surface.

- Massive sulphide (50–75%) confirmed at Berry Shaft in 2024 surface grab.

## What has not been done

- No systematic modern diamond drill campaign has ever been executed on the Offset Dyke target at depth (300–1,000 m).

- No modern channel sampling with QA/QC protocols in the Alanen or Berry Shaft trenches.

- The VTEM anomaly identified prior to 2022 remains unexplained — its conductor source has not been located.

- The ground EM survey recommended by Vale in 2022 (600+ line-metres) was never completed.

- Systematic soil geochemistry and lithogeochemistry across the full 36 km² block has not been completed.

## Exploration Risk Assessment

The principal risk at Nairn is geological — the SOD must be present and sufficiently mineralised at depth. The risk-mitigation path is straightforward: modern IP/EM geophysics followed by a focused first-pass drill campaign. The surface evidence is compelling; the question is depth. Given the Sudbury precedent (grades increasing with depth in all comparable SOD systems), the Nairn program represents a well-defined, binary exploration decision with a clear positive outcome if the depth target is confirmed.

ASSESSMENT FILE REFERENCES

GeologyOntario Assessment Files: IDs 20000022000, 20000022112, 20000021235, 20000008521. All reports available via GeologyOntario online portal (https://www.geologyontario.mndm.gov.on.ca).